A Comprehensive Review on Modeling, Design and Control of Renewable Energy-Based Microgrids

Sanket Kolhe, Arpita Bhalekar, Vishal Bhale, Pallavi Jumbad, Prof. H.R Shinde

Department of Electrical Engineering, Adsul’s Technical Campus, Ahilyanagar Savitribai Phule Pune University

Abstract— This review summarizes the major modeling, design, and control techniques used in hybrid microgrid systems that combine solar photovoltaic (PV) and wind energy sources. Hybrid renewable microgrids play an important role in handling the intermittency of clean energy and improving overall system reliability, as noted in recent studies on hybrid energy integration [1], [22]. The review covers the mathematical modeling of PV modules [32], wind systems based on PMSG generators [7], and commonly used MPPT algorithms that help extract maximum available power under changing environmental conditions [11], [12]. The function of DC–DC converters and Voltage Source Inverters (VSI) in voltage regulation, power conversion, and grid synchronization is discussed with reference to recent advancements in inverter control [4], [5]. System parameters such as voltage stability, harmonic mitigation, and frequency regulation, which are widely studied in microgrid control literature, are also addressed [9], [26]. MATLAB/Simulink-based simulations, which are commonly used for evaluating microgrid performance [36], are considered for analyzing dynamic behavior under varying irradiance, wind speed, and load conditions. Overall, the review highlights that coordinated control and efficient power management significantly improve the reliability, quality, and operational flexibility of hybrid renewable microgrids, supporting their integration into future smart-grid structures [21], [27].

Keywords— Renewable Energy, Microgrid, Solar Photovoltaic (PV), Wind Energy, Maximum Power Point Tracking (MPPT), DC–DC Converter, Voltage Source Inverter (VSI), MATLAB/Simulink, Hybrid Energy System, Grid Integration, Power Quality, PMSG, Control Strategies.

I. INTRODUCTION

The global shift toward clean and sustainable energy has increased the demand for power systems that rely on renewable sources rather than traditional fossil fuels. Among these alternatives, solar photovoltaic (PV) and wind energy have gained major importance because of their wide availability, environmental benefits, and technological progress over recent years [22], [27]. However, both resources fluctuate with weather conditions, resulting in unstable voltage, frequency deviations, and variations in power quality. These limitations have encouraged the development of hybrid renewable microgrids, which integrate multiple distributed energy resources (DERs) to improve reliability and support continuous power supply [1], [23]. A microgrid is a small-scale energy network capable of operating in both grid-connected and islanded modes, allowing flexible operation and enhanced resilience [21].

In hybrid PV–wind microgrids, power electronic devices play a crucial role in energy conversion and system stability. MPPT algorithms such as Perturb and Observe (P&O) ensure that PV and wind systems operate near their maximum power point under changing environmental conditions [11], [12]. DC–DC converters help regulate voltage at the DC link, while a Voltage Source Inverter (VSI) enables AC power generation, grid synchronization, and improvement in power quality [4], [5], [7]. Intelligent control methods, including model predictive and adaptive control, are widely explored to minimize harmonics, manage dynamic fluctuations, and maintain system stability [9], [26]. Simulation tools such as MATLAB/Simulink provide a practical platform for analyzing the dynamic performance of microgrids under different operating conditions [36].

This paper presents a complete review of the modeling, design, and control techniques for a hybrid microgrid that integrates solar and wind energy. It includes the mathematical modeling of each subsystem, MPPT strategies, converter design, inverter control, and energy management approaches. The goal is to demonstrate how coordinated control and system integration can enhance reliability, sustainability, and power quality, contributing to next-generation smart grid developments [27], [33].

# II. Background and Key Concepts

The rapid adoption of renewable energy technologies has led to major changes in modern power systems, moving them from centralized fossil-fuel-based generation to distributed and environmentally friendly renewable sources. Solar photovoltaic (PV) and wind energy are among the most widely used options because of their technological maturity and long-term economic viability [22], [23]. However, these energy sources depend heavily on weather conditions, which causes fluctuations in output, voltage instability, and variations in power quality. To overcome such limitations, hybrid systems that combine multiple renewable sources are increasingly being used to ensure consistent and reliable electricity generation [1], [27].

A microgrid is a small, self-contained power network that includes distributed energy resources (DERs), energy storage systems, and local loads. It can function in both grid-connected and islanded modes, providing improved reliability, reduced transmission losses, and enhanced energy security [21]. In a hybrid microgrid, solar and wind systems complement each other, as PV output is typically higher during daytime hours while wind generation may peak during evenings or different seasons. This complementary nature helps reduce power variability and improve the overall stability of the system [22], [36].

The operation of hybrid microgrids depends on several power electronic components and control strategies. PV systems are typically modeled using single-diode or multi-diode equivalent circuits, which describe their electrical behavior under varying irradiance and temperature [32]. To ensure maximum power extraction, MPPT algorithms such as Perturb and Observe (P&O), Incremental Conductance, and advanced fast-tracking methods are widely used [11], [12]. Wind energy conversion systems often use Permanent Magnet Synchronous Generators (PMSG) due to their higher efficiency, reduced maintenance, and suitability for variable-speed operation [7], [24]. Both PV and wind outputs are interfaced through DC–DC converters that regulate the DC link voltage before the power is converted to AC using a Voltage Source Inverter (VSI) [5], [8].

Key performance indicators in hybrid microgrids include voltage stability, frequency regulation, total harmonic distortion (THD), and effective power sharing among multiple sources. These factors depend heavily on inverter control strategies such as model predictive control (MPC), adaptive control, and vector control, which help improve system dynamics and power quality [4], [9], [26]. Additionally, an Energy Management System (EMS) ensures proper coordination between renewable sources, storage units, and local loads to maintain balanced and reliable operation [21].

# These foundational concepts form the basis for understanding hybrid microgrid modeling, analysis, and control, and they serve as the groundwork for designing future smart and resilient power systems.

# III. Literature Review

Research on renewable-energy-based hybrid microgrids has expanded significantly in recent years, with several studies focusing on improving reliability, control performance, and energy management. Fang et al. [1] presented an extensive review of hybrid energy storage systems (HESS), highlighting their importance in smoothing renewable power fluctuations and maintaining system stability. Similar findings were reported by Urooj [2], who emphasized the benefits of combining high-power and high-energy storage technologies to optimize renewable energy utilization. Lucas et al. [3] introduced a sizing model for hybrid storage systems, demonstrating that proper sizing and control directly influence load management and microgrid efficiency.

Control strategies for inverters in renewable integrated systems have also been widely investigated. Wen et al. [4] proposed an MPC-based control strategy for PV grid-connected inverters, showing significant reductions in power oscillations and improved dynamic performance. Muhammad et al. [5] developed an adaptive hybrid control method that enhances inverter robustness under changing grid impedance, reducing Total Harmonic Distortion (THD). Meng et al. [6] further explored adaptive model predictive control techniques for grid-forming converters, enabling smoother transitions between islanded and grid-connected modes. Studies by Hu et al. [7] and Tang et al. [8] also demonstrate that predictive control and Kalman filter-based approaches significantly enhance grid stability under unbalanced network conditions.

Accurate modeling of renewable energy systems is essential for microgrid design and simulation. Sera et al. [32] developed a widely used PV model based on manufacturer datasheet parameters, facilitating efficient analysis and MPPT design. For wind systems, Permanent Magnet Synchronous Generators (PMSGs) have been studied extensively due to their suitability for variable-speed operation and high efficiency [24]. Various MPPT techniques, including P&O, Tip-Speed Ratio (TSR), and advanced adaptive methods, have been evaluated in recent literature, with reviews from Katche et al. [11], Liu [12], and Ali [13] providing comprehensive comparisons of tracking accuracy and dynamic performance.

Power quality and inverter control represent another major focus area in hybrid microgrid research. Mirza and Bollen [9] reviewed recent developments in finite-control-set MPC for grid-connected converters, highlighting improvements in THD and dynamic response. Studies by Geng [10], Mirza [28], Franquelo and Rodriguez [30], and others provide valuable insights into multi-level inverter topologies, two-vector MPC, and vector preselection methods for enhancing power quality. Energy management strategies are also addressed in multiple reviews, including the work of Zhu et al. [21], which emphasizes the importance of hierarchical EMS frameworks for coordinating renewable generation, storage, and load demand.

Overall, existing research clearly shows that hybrid solar–wind microgrids, supported by advanced control, accurate modeling, and efficient energy management, offer strong potential for improving power quality, stability, and sustainability. However, further developments in intelligent control, optimization, and real-time energy management are needed to address remaining challenges and support the growing integration of renewables into modern power systems.

IV. Methodology

The methodology adopted in this study focuses on the structured modeling, design, and performance evaluation of a hybrid microgrid that integrates solar photovoltaic (PV) and wind energy systems. The PV subsystem is modeled using a single-diode equivalent circuit, which is widely used for analyzing PV characteristics under varying temperature and irradiance conditions [32]. To ensure maximum power extraction, the Perturb and Observe (P&O) MPPT algorithm is implemented due to its simplicity, reliability, and proven effectiveness across different operating conditions [11]. The wind subsystem is modeled based on turbine aerodynamic principles and is paired with a Permanent Magnet Synchronous Generator (PMSG), which offers high efficiency and reliable variable-speed operation [7], [24]. MPPT for the wind system is achieved by analyzing turbine power–speed characteristics and controlling the tip-speed ratio to maintain optimal energy conversion.

Both renewable sources are interfaced with a common DC link through DC–DC converters, which help regulate the DC voltage and ensure stable power transfer to subsequent stages [8]. A Voltage Source Inverter (VSI) is designed to convert the regulated DC power into AC power suitable for grid integration. The inverter operates using a synchronous reference frame–based control method, enabling precise control of active and reactive power, reduced harmonic distortion, and improved grid synchronization [4], [5]. Advanced predictive control concepts from recent literature also guide the selection of appropriate control parameters to enhance dynamic response and system stability [9], [26].

An Energy Management System (EMS) coordinates the power flow between the PV system, wind generator, battery storage, and the grid. The EMS ensures that the system balances supply and demand, maintains the DC link voltage within safe limits, and improves overall operational reliability, aligning with energy management principles discussed in recent reviews [21].

The entire system is modeled and simulated using MATLAB/Simulink, which provides an effective environment for analyzing hybrid microgrid behavior under different irradiance levels, wind speeds, and load variations [36]. Performance metrics such as voltage stability, power quality, MPPT efficiency, and dynamic response are evaluated to validate the effectiveness of the hybrid microgrid design.

V.  Discussion

The analysis of the proposed hybrid microgrid highlights the importance of coordinated control, accurate subsystem modeling, and efficient energy management in renewable-power-based systems. The complementary operation of solar PV and wind energy helps minimize the impact of intermittency, resulting in more stable and continuous power generation compared to using a single renewable source [22], [27]. The application of MPPT algorithms plays a crucial role in maintaining optimal operating points for both PV and wind systems. The P&O method used for PV and TSR-based MPPT for wind systems allow effective adaptation to changes in irradiance and wind speed, supporting consistent maximum power extraction [11], [12].

The performance of the overall hybrid microgrid significantly depends on the behavior of power electronic converters and inverter control strategies. DC–DC converters help stabilize the DC link voltage and ensure smooth energy transfer from renewable sources to the inverter stage [8]. On the other hand, the VSI improves power quality, maintains voltage and frequency stability, and ensures proper synchronization with the grid, as supported by various studies on predictive and adaptive control methods [4], [5], [7]. These advanced control strategies contribute to reduced Total Harmonic Distortion (THD), faster transient response, and enhanced robustness under varying network conditions [9], [26].

The integration of battery storage provides an additional layer of stability by compensating for sudden fluctuations in renewable generation and managing short-term load variations. This aligns with findings in the literature that highlight the importance of hybrid storage systems for maintaining microgrid reliability and improving power balancing [1], [3]. The Energy Management System (EMS) facilitates the coordination of renewable energy sources, storage units, and loads, ensuring efficient utilization of available power and avoiding system-level conflicts [21].

Simulation results—obtained through MATLAB/Simulink, a widely used tool for microgrid research [36]—indicate notable improvements in voltage regulation, dynamic response, and overall operational performance. These findings demonstrate that hybrid microgrids can effectively support cleaner and more reliable power supply. However, challenges remain, including the dependency on weather conditions, increasing system complexity due to multiple control loops, and the need for real-time optimization techniques. Future advancements in artificial intelligence, predictive control, and digital-twin-based monitoring systems show strong potential for addressing these challenges and further enhancing microgrid performance [24], [26].

VI. Future Scope

The hybrid renewable microgrid presented in this study offers several opportunities for further improvement through advancements in control, optimization, and system integration. One major direction for future work is the application of artificial intelligence (AI)-based algorithms—including machine learning, deep reinforcement learning, and predictive modeling—to enhance MPPT performance, load forecasting, and dynamic energy management [14], [15]. These techniques have shown strong potential in improving adaptability and decision-making in complex microgrid environments. Another promising area is the implementation of digital twin technology, which enables real-time system monitoring, fault diagnosis, and predictive maintenance, thereby improving reliability and reducing downtime [24].

Future microgrid designs can also integrate additional renewable sources such as biomass, fuel cells, or micro-hydro power systems to create a more diversified and resilient multi-source architecture [27], [35]. Enhancing the energy storage portion of the system by adopting hybrid storage technologies—such as combinations of batteries, supercapacitors, and hydrogen storage—may further improve transient response, reduce battery stress, and extend operational lifespan [1], [25].

On the hardware and control side, next-generation grid-forming inverters and advanced MPC-based control strategies are expected to significantly improve system stability, especially during disturbances or transitions between grid-connected and islanded modes [17], [30]. Additionally, large-scale experimental validation, hardware-in-the-loop (HIL) testing, and real-time simulation platforms can be used to evaluate system performance and accelerate the transition of microgrid research from simulation to practical deployment [41], [42].

Overall, the hybrid microgrid model can evolve into a more intelligent, scalable, and adaptive energy system capable of supporting future smart-grid infrastructures and large-scale renewable integration.

VII. CONCLUSION

This study presents a detailed review of the modeling, design, and control aspects of a hybrid microgrid that integrates solar photovoltaic (PV) and wind energy systems. The complementary characteristics of these renewable sources allow for more reliable and stable power generation, helping to minimize the impact of intermittency and environmental variations [22], [27]. The use of MPPT techniques—such as P&O for PV and TSR-based methods for wind—ensures efficient power extraction under varying operating conditions [11], [12]. DC–DC converters effectively regulate the DC link voltage, while the Voltage Source Inverter (VSI) maintains grid synchronization, enhances power quality, and supports stable AC output, as shown in multiple recent studies on inverter control [4], [5], [7].

Simulation results obtained through MATLAB/Simulink, a widely adopted tool for analyzing renewable-integrated systems [36], confirm improvements in voltage stability, frequency regulation, harmonic reduction, and dynamic response. The role of the Energy Management System (EMS) is crucial, as it coordinates the power flow between renewable sources, storage units, and loads to maintain reliable microgrid operation [21]. Overall, the findings indicate that well-designed hybrid renewable microgrids can significantly contribute to building sustainable, efficient, and resilient power networks.

Furthermore, advancements in intelligent control, hybrid energy storage, and next-generation inverter technologies offer promising pathways for future improvement. With continued research and technological innovation, hybrid microgrids will play a key role in supporting widespread renewable energy integration and the development of future smart-grid infrastructures [1], [24], [26].

REFERENCES

[1] Z. Fang, J. K. H. Shek and W. Sun, “A review of grid-connected hybrid energy storage systems: sizing, configurations, control strategies and future directions,” Renewable Energy Reviews, vol. 118, Art. 116226, May 2025.

[2] A. Urooj, “Review of hybrid energy storage systems for renewable applications,” Energies, vol. 15, no. 8, Art. 342, 2024.

[3] A. Lucas, S. Golmaryami and S. Carvalhosa, “Hybrid energy storage system sizing model based on load recurring pattern identification,” Energy Reports, vol. 11, pp. 1213–1224, 2024.

[4] X. Wen, T. Wu, H. Jiang, J. Peng and H. Wang, “MPC-based control strategy of PV grid connected inverter for damping power oscillations,” Frontiers in Energy Research, vol. 10, Art. 968910, Sep. 2022.

[5] T. Muhammad et al., “An adaptive hybrid control of grid-tied inverter for the reduction of total harmonic distortion and improvement of robustness against grid impedance variation,” Energies, vol. 15, no. 13, Art. 4724, Jun. 2022.

[6] J. Meng et al., “Adaptive model predictive control for grid-forming converters to achieve smooth transition from islanded to grid-connected mode,” IET Generation, Transmission & Distribution, vol. 17, no. 12, pp. 2833–2845, May 2023.

[7] B. Hu, Z. Chen, Z. Zhang, S. Liu and W. Deng, “Model predictive flexible power control for grid-connected inverter under unbalanced network,” IET Power Electronics, vol. 13, no. 16, pp. 3580–3589, Dec. 2020.

[8] X. Tang, Z. Zhang and B. Hu, “Model predictive power control with Kalman filter for grid-tied inverter with parameter variation,” IET Power Electronics, vol. 15, no. 7, pp. 621–630, 2022.

[9] S. Mirza and M. H. J. Bollen, “New approaches in finite control set model predictive control for grid-connected converters: a review,” Energies, Special Issue, 2024.

[10] Y. Geng, “Model predictive control of three-phase grid-connected inverter under unbalanced and harmonic grids,” IET Power Electronics, 2023.

[11] M. L. Katche, A. B. Makokha, S. O. Zachary and M. S. Adaramola, “A comprehensive review of maximum power point tracking (MPPT) techniques used in solar PV systems,” Energies, vol. 16, 2023.

[12] Y. H. Liu, “Comprehensive review on fast maximum power point tracking methods,” Renewable Energy, 2024.

[13] M. H. Ali, “A comprehensive study of recent maximum power point tracking techniques and performance ranking,” Scientific Reports, 2025.

[14] S. C. Das, “Deep reinforcement learning for inverter control: adaptive gain tuning strategies for power system stability,” arXiv preprint, Nov. 2024.

[15] F. Zhao, “Deep data-enabled policy optimization for grid-connected converters,” arXiv preprint, Nov. 2024.

[16] R. Pudur, “A comprehensive review of grid-connected inverter technologies (2020–2025),” Energy, Elsevier Review, 2025.

[17] S. Yusuf, “A comprehensive review on grid-forming inverter,” Modern Journal of Electrical Engineering, 2023.

[18] C. Li, “Modeling, control and stabilization of virtual synchronous generators (VSGs),” Renewable & Sustainable Energy Reviews, 2025.

[19] N. Baeckeland, “Overcurrent limiting in grid-forming inverters,” NREL Technical Report, 2024.

[20] IEEE PES, “Grid-Forming Inverter-Based Resource Research Landscape,” IEEE-PES Whitepaper, Mar. 2024.

[21] R. Zhu, K. Das, P. E. Sørensen and A. D. Hansen, “A review on energy management system for grid-connected utility-scale renewable hybrid power plants,” Wiley Interdisciplinary Reviews: Energy and Environment, 2025.

[22] A. Khan, “Comprehensive review of hybrid energy systems,” Energies, vol. 18, no. 10, Art. 2612, 2025.

[23] A. Giedraityte, “Hybrid renewable energy systems—balancing economic, environmental, social and technical criteria,” Energies, 2025.

[24] M. Z. Yameen, “Improving frequency stability in grid-forming inverters with adaptive model predictive control,” Scientific Reports, 2025.

[25] M. Bareev-Rudy, “Sizing optimization of hybrid hydrogen energy storage for residential scenarios,” Applied Energy, Elsevier, 2025.

[26] K. Boutaghane et al., “New control strategies to improve effectiveness of multi-source hybrid energy systems,” Scientific Reports, 2025.

[27] J. C. León Gómez, “A review of hybrid renewable energy systems,” Energies, 2023.

[28] S. Mirza, “Finite-control-set MPC and two-vector strategies for PV inverters: survey and implementations,” Energies, Special Issue, 2024.

[29] R. Camacho and C. Bordons, Model Predictive Control, 2nd ed., Springer, 2007.

[30] L. G. Franquelo, J. Rodriguez, “Finite-control-set MPC for three-phase voltage source inverters,” IEEE Trans. Ind. Electron., 2007–2010.

[31] J. Rodriguez, J. Lai and F. Z. Peng, “Multilevel inverters: a survey of topologies, controls and applications,” IEEE Trans. Ind. Electron., 2002.

[32] D. Sera, R. Teodorescu and P. Rodriguez, “PV panel model based on datasheet values,” IEEE Trans. Ind. Electron., 2013.

[33] N. Jenkins, “Integration of large-scale renewables and HESS for system stability: review and challenges,” Renewable Energy, 2022.

[34] P. Lund, “Hybrid energy storage: batteries + supercapacitors case studies and controllers,” Journal of Energy Storage, 2023.

[35] S. Vendoti et al., “Grid-tied hybrid PV–fuel cell system with energy storage: design and experimental validation,” Scientific Reports, 2025.

[36] W. F. Mbasso, “Reliability analysis of grid-connected hybrid renewable energy systems,” Frontiers in Energy Research, 2024.

[37] H. Z. Al Garni, “Comprehensive review of MPPT techniques for PV under partial shading,” Journal of Renewable Energy Engineering, 2024.

[38] Y. Geng and X. Zhu, “FCS-MPC for three-phase PV inverter with low switching loss,” IET Power Electronics, 2021–2023.

[39] S. Mirza et al., “Two-vector FCS-MPC with vector preselection for reduced switching and THD,” Energies, 2022–2024.

[40] B. Singh and S. Goel, “Hybrid MPC-adaptive control for grid-tied PV inverters: survey and comparative results,” Energies, 2024.

[41] J. Meng et al., “Hardware-in-the-loop validation methods for MPC in grid-forming converters,” IEEE Trans., 2023.

[42] NREL/IEEE, “Practical considerations for embedded MPC and HIL testing for inverter control,” 2022–2024.